ITELECTIVE 3 – INTERNET OF THINGS LAB

Laboratory Activity 2: Arduino LED Traffic Light Sequence

| Student Name: | ________________________________________ |
| --- | --- |
| Program / Year & Section: | ________________________________________ |
| Date: | ________________________________________ |
| Score: | ________________________________________ |

I. Activity Title

Arduino LED Traffic Light Sequence: Introduction to Multiple Digital Outputs

II. Related Course Topics

Arduino; GPIO; Arduino IDE; Digital Output; Embedded Programming; Program Logic; Functions; Timing

III. Learning Objectives

Identify the basic components needed to create a three-LED traffic light circuit.

Use multiple Arduino digital output pins to control LEDs independently.

Understand and apply pinMode(), digitalWrite(), delay(), and user-defined functions.

Program LEDs to operate in a traffic-light sequence using program logic and timing.

Modify timing values and create a customized LED sequence.

IV. Progression from Activity 1

| Activity 1 | Activity 2 | Student Practice |
| --- | --- | --- |
| 1 LED – ON/OFF | 3 LEDs – sequential control | Multiple GPIO pins |
| Basic digital output | Traffic-light sequence | Functions and timing |
| pinMode(), digitalWrite(), delay() | Program logic and sequencing | Modify and create sequences |

V. Materials / Equipment

Arduino Uno or compatible Arduino board

USB cable

Breadboard

1 Red LED

1 Yellow LED

1 Green LED

3 × 220 Ω or 330 Ω resistors

Jumper wires

Computer with Arduino IDE installed

VI. Circuit Connection / Guide

Build the circuit using three separate digital output pins. Each LED must have its own current-limiting resistor.

| Arduino Pin | Component | Connection |
| --- | --- | --- |
| Pin 10 | Red LED | Pin 10 → 220/330 Ω resistor → Red LED anode (long leg); cathode (short leg) → GND |
| Pin 9 | Yellow LED | Pin 9 → 220/330 Ω resistor → Yellow LED anode (long leg); cathode (short leg) → GND |
| Pin 8 | Green LED | Pin 8 → 220/330 Ω resistor → Green LED anode (long leg); cathode (short leg) → GND |

Connection layout (text guide):

Arduino GND ───────── Breadboard GND rail
Pin 10 ──[220/330 Ω]──► RED LED ──► GND
Pin 9  ──[220/330 Ω]──► YELLOW LED ──► GND
Pin 8  ──[220/330 Ω]──► GREEN LED ──► GND

Safety note: The resistor is required for each external LED to limit current and help protect the LED and Arduino output pin.

VII. Expected Traffic Light Sequence

| Step | Red | Yellow | Green |
| --- | --- | --- | --- |
| 1 | ON | OFF | OFF |
| 2 | OFF | OFF | ON |
| 3 | OFF | ON | OFF |

Default timing: Red = 3 seconds, Green = 3 seconds, Yellow = 1 second. The sequence repeats continuously.

IX. Arduino Program

// Arduino LED Traffic Light Activity 2

( -- Your Code Here -- )

X. Code Explanation

| Command / Function | Purpose |
| --- | --- |
| const int redLED = 10; | Assigns Arduino pin 10 to the red LED. |
| pinMode(pin, OUTPUT); | Sets each LED pin as a digital output. |
| digitalWrite(pin, HIGH); | Turns the selected LED ON. |
| digitalWrite(pin, LOW); | Turns the selected LED OFF. |
| delay(3000); | Pauses the program for 3000 ms (3 seconds). |
| allOff(); | Turns all three LEDs OFF before the next state. |
| redLight(), greenLight(), yellowLight() | User-defined functions that make each traffic-light state easier to read and reuse. |
| setup() | Runs once when the Arduino starts. |
| loop() | Runs repeatedly while the Arduino is powered. |

XI. Challenge Tasks

Challenge 1: Change the Red, Green, and Yellow timing values to 5 seconds, 4 seconds, and 2 seconds respectively.

Challenge 2: Create a flashing-yellow mode after the normal sequence. Make the yellow LED blink 3 times before returning to Red.

Challenge 3: Create a pedestrian-style sequence: Green → Yellow → Red, with your own timing values.

Challenge 4: Create your own traffic-light sequence using the three LEDs. Explain the logic and timing you selected.

Challenge 5 (Optional): Add a fourth LED or another output device and integrate it into your sequence.

XII. Required Output / Submission

Screenshot or photo of the actual three-LED circuit.

Screenshot of the Arduino code in the Arduino IDE.

Completed observation table and answers to the guide questions.

Completed challenge task(s).

Link of your thinkerCAD activity

XIII. Observation Table

| Test | Red Time | Green Time | Yellow Time | Observation |
| --- | --- | --- | --- | --- |
| Original Program |  |  |  |  |
| Challenge 1 |  |  |  |  |
| Challenge 2 |  |  |  |  |
| Challenge 3 |  |  |  |  |
| Challenge 4 |  |  |  |  |